BASIC SPIDER ROBOT
Arduino Uno + PCA9685 + 8-Servo Quadruped

Sesame-style basic quadruped with obstacle avoidance and expressive OLED face

# 1. Project Overview

This project is a basic four-legged (quadruped) spider robot based on the Sesame-style design reference. It uses an Arduino Uno as the main controller, a PCA9685 16-channel servo driver for eight MG90S servos, a 128×64 OLED for facial expressions, and VL53L0X + HC-SR04 sensors for obstacle awareness.

# 2. Features

Four-leg walking using 8 servos (2 servos per leg).

Forward and backward walking.

Left/right movement and turning through programmed gait sequences.

Standing and programmed poses.

Head movement when the selected mechanical design includes a head servo.

Close-range obstacle detection using VL53L0X ToF.

Longer-range obstacle detection using HC-SR04.

Automatic stop/turn behavior when an obstacle is detected.

128×64 OLED animated facial expressions.

Normal, happy, sad, angry, surprised, sleepy, scanning and obstacle-warning expressions.

Buzzer feedback.

Programmable servo positions and walking frames.

# 3. Required Electronics Components

| No. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Arduino Uno R3 | ATmega328P, 5 V | 1 | Main controller |
| 2 | PCA9685 Servo Driver | 16-channel, I²C, 12-bit PWM | 1 | Controls 8 servos |
| 3 | MG90S Servo Motor | Metal gear, ~180° | 8 | Two servos per leg |
| 4 | 128×64 OLED Display | 0.96-inch, I²C | 1 | Face and expressions |
| 5 | VL53L0X ToF Sensor | I²C time-of-flight | 1 | Close-range obstacle detection |
| 6 | HC-SR04 Ultrasonic Sensor | 40 kHz | 1 | Longer-range obstacle detection |
| 7 | 5–6 V High-Current BEC/Buck Converter | Sized for servo load | 1 | Servo power |
| 8 | Rechargeable Battery Pack | Suitable voltage/capacity | 1 | Main power |
| 9 | Power Switch | SPST | 1 | Main ON/OFF |
| 10 | 470 µF Electrolytic Capacitor | 10–16 V or suitable rating | 1 | Servo supply smoothing |
| 11 | 5 V Buzzer | Active/suitable buzzer | 1 | Sound/alerts |
| 12 | Servo Extension Cables | 3-pin | 8 | Servo connections |
| 13 | Jumper Wire Set | M-M / M-F / F-F | 1 set | Connections |

# 4. Required Mechanical Components

| Component | Qty | Purpose |
| --- | --- | --- |
| 3D-printed main body | 1 set | Main chassis |
| 3D-printed head | 1 | Display/sensor enclosure |
| 3D-printed leg assemblies | 4 sets | Four walking legs |
| Servo mounting brackets | 8 | Mount servos |
| Servo horns | 8 | Connect servos to joints |
| Robot feet | 4 | Ground contact |
| M2/M3 screws and nuts | 1 set | Assembly |
| M2/M3 washers | 1 set | Fastening |
| Spacers/standoffs | 1 set | PCB mounting |
| Cable ties/mounting tape | 1 set | Secure wiring |

# 5. Servo and Leg Configuration

The basic quadruped uses 8 MG90S servos: two servos per leg. The exact servo numbering should follow the mechanical design and movement software.

Front-left leg: 2 servos

Front-right leg: 2 servos

Rear-left leg: 2 servos

Rear-right leg: 2 servos

# 6. Electrical Architecture

Arduino Uno communicates with the PCA9685 over I²C. The PCA9685 generates the PWM signals for the eight servos. The servos receive power from a separate high-current 5–6 V supply. The Arduino must not supply the servo current.

The VL53L0X and OLED can share the Arduino Uno I²C bus: SDA = A4 and SCL = A5. The HC-SR04 uses one trigger output and one echo input. The buzzer uses one digital output.

# 7. Suggested Connections

| Device | Connection | Function |
| --- | --- | --- |
| PCA9685 VCC | Arduino 5V | Logic power |
| PCA9685 GND | Arduino GND | Common ground |
| PCA9685 SDA | A4 | I²C data |
| PCA9685 SCL | A5 | I²C clock |
| PCA9685 V+ | External regulated 5–6 V | Servo power |
| MG90S servos | PCA9685 channels 0–7 | Servo outputs |
| VL53L0X SDA | A4 | I²C data |
| VL53L0X SCL | A5 | I²C clock |
| VL53L0X GND | GND | Ground |
| VL53L0X VIN | According to breakout specification | Sensor power |
| OLED SDA | A4 | I²C data |
| OLED SCL | A5 | I²C clock |
| OLED GND | GND | Ground |
| OLED VCC | According to display specification | Display power |
| HC-SR04 TRIG | Arduino digital output | Trigger |
| HC-SR04 ECHO | Arduino digital input | Echo |
| Buzzer + | Arduino digital output (transistor driver if needed) | Sound |
| Buzzer − | GND | Return |

# 8. Power System

Use a battery and regulator/BEC appropriate for the selected servo load. The servo supply must handle the combined current demanded when several servos move. Connect servo-supply ground, PCA9685 ground and Arduino ground together.

Place the 470 µF electrolytic capacitor across the servo V+ and GND near the PCA9685. Observe polarity: positive to V+, negative to GND.

# 9. Obstacle Avoidance

The HC-SR04 and VL53L0X provide complementary measurements. The HC-SR04 is used for longer-range detection; the VL53L0X provides precise close-range measurement.

Clear path → continue walking.

Obstacle detected → prepare to stop/turn.

Obstacle close → stop and execute an avoidance gait.

Path clear → resume walking.

# 10. OLED Expression System

Normal/idle

Happy

Sad

Angry

Surprised

Sleepy

Thinking/scanning

Obstacle warning

Walking animation

Expressions can be synchronized with movement frames. For example, show a surprised/warning face when an obstacle is detected.

# 11. Movement Features

Stand

Forward

Backward

Left turn

Right turn

Stop

Programmable poses

Obstacle-avoidance gait

# 12. Not Required in the Basic Version

L293D motor driver

L298N motor driver

MPU6050 IMU

DC/BO motors

Camera

ESP32

Additional stabilization sensors

L293D and L298N are DC-motor drivers and are not used for the MG90S servo legs. PCA9685 is the servo-control interface in this design.

# 13. Safety and Design Notes

Do not power eight MG90S servos from the Arduino Uno 5 V pin.

Use a separate, adequately rated 5–6 V servo BEC/power supply.

Keep Arduino, PCA9685 and servo-supply grounds common.

Verify the exact VL53L0X and OLED breakout-board voltage requirements before powering them.

Check the HC-SR04 module and Arduino input arrangement before wiring; use suitable level protection if required by the controller/module.

The 470 µF capacitor helps stabilize the servo supply but does not replace a correctly sized power supply.

Do not force servos against mechanical stops.

Secure the battery and wiring away from moving legs.

# 14. Final Bill of Materials

| Component | Qty | Category |
| --- | --- | --- |
| Arduino Uno R3 | 1 | Electronics |
| PCA9685 16-channel servo driver | 1 | Electronics |
| MG90S servo motor | 8 | Electronics |
| 128×64 I²C OLED | 1 | Electronics |
| VL53L0X ToF sensor | 1 | Electronics |
| HC-SR04 ultrasonic sensor | 1 | Electronics |
| 5–6 V high-current BEC/buck converter | 1 | Power |
| Rechargeable battery pack | 1 | Power |
| SPST power switch | 1 | Power |
| 470 µF electrolytic capacitor | 1 | Power |
| 5 V buzzer | 1 | Electronics |
| 3-pin servo extension cables | 8 | Wiring |
| Jumper wire set | 1 | Wiring |
| 3D-printed body/head/legs | 1 | Mechanical |
| Servo mounting brackets | 8 | Mechanical |
| Servo horns | 8 | Mechanical |
| Robot feet | 4 | Mechanical |
| M2/M3 screws, nuts and washers | 1 | Mechanical |
| Spacers/standoffs | 1 | Mechanical |
| Cable ties/mounting materials | 1 | Mechanical |

Final configuration: Arduino Uno • PCA9685 • 8 MG90S • OLED • VL53L0X • HC-SR04