BASIC SPIDER ROBOT
Component List, Features & Sensor Functions

Arduino Uno based Sesame-style quadruped robot

# 1. Project Description

The Basic Spider Robot is a four-legged quadruped robot based on the Sesame-style design. The basic version uses an Arduino Uno, eight MG90S servo motors, a PCA9685 servo driver, an OLED display for expressions, and two types of distance sensors for obstacle awareness. The design focuses on walking, simple head movement, obstacle avoidance, and expressive behavior.

# 2. Required Components

| No. | Component | Specification | Qty | Purpose |
| --- | --- | --- | --- | --- |
| 1 | Arduino Uno R3 | ATmega328P | 1 | Main controller for movement, sensors and expressions |
| 2 | PCA9685 Servo Driver | 16-channel I²C, 12-bit PWM | 1 | Provides PWM control for the eight leg servos |
| 3 | MG90S Servo Motor | Metal gear, approximately 180° | 8 | Two servos per leg for quadruped movement |
| 4 | 128×64 OLED Display | 0.96-inch, I²C | 1 | Displays the robot's eyes and expressions |
| 5 | VL53L0X ToF Sensor | Infrared Time-of-Flight | 1 | Precise short-range obstacle/distance detection |
| 6 | HC-SR04 Ultrasonic Sensor | 40 kHz ultrasonic | 1 | Longer-range obstacle detection |
| 7 | 5–6 V High-Current BEC/Buck Converter | Adequate for servo load | 1 | Provides suitable regulated servo power |
| 8 | Rechargeable Battery Pack | Suitable voltage/capacity | 1 | Portable power source |
| 9 | SPST Power Switch | DC-rated | 1 | Main power ON/OFF |
| 10 | 470 µF Electrolytic Capacitor | 10–16 V or suitable rating | 1 | Helps stabilize the servo power supply |
| 11 | 5 V Buzzer | Active/suitable buzzer | 1 | Audio feedback and alerts |
| 12 | 3D-Printed Robot Body | Sesame-style chassis | 1 set | Main mechanical structure |
| 13 | 3D-Printed Head | Designed for OLED/sensors | 1 | Robot head and face housing |
| 14 | 3D-Printed Leg Assemblies | Four-leg quadruped parts | 4 sets | Mechanical walking structure |
| 15 | Servo Mounting Brackets | Compatible with MG90S | 8 | Mounts the servos to the legs/body |
| 16 | Servo Horns | MG90S compatible | 8 | Transfers servo rotation to leg joints |
| 17 | Robot Feet | Compatible with leg design | 4 | Ground contact and stability |
| 18 | M2/M3 Screws, Nuts & Washers | Mechanical hardware set | 1 set | Assembles the robot |
| 19 | Spacers/Standoffs | M2/M3 compatible | 1 set | Mounts electronics inside the body |

# 3. Why the Sensors Are Used

## VL53L0X ToF Sensor

The VL53L0X measures distance using infrared Time-of-Flight technology. It is used mainly for close-range and more precise obstacle detection. It is compact and suitable for mounting on the front of the robot head.

Detects nearby obstacles before the robot reaches them.

Provides a direct distance measurement rather than only an obstacle/no-obstacle signal.

Useful for close-range automatic stopping and turning.

Small size makes it suitable for the robot head.

## HC-SR04 Ultrasonic Sensor

The HC-SR04 measures distance by transmitting an ultrasonic pulse and measuring its echo. It is used to provide longer-range obstacle awareness in front of the robot.

Detects obstacles at a greater distance than the close-range VL53L0X use case.

Provides distance information that can be used to prepare the robot for an upcoming turn.

Useful as a second, independent obstacle-detection method.

## Why Use Both Sensors?

The two sensors complement each other. The HC-SR04 provides longer-range awareness, while the VL53L0X provides precise close-range measurement. Together they give the robot a two-stage obstacle detection system.

| Situation | Sensor | Robot response |
| --- | --- | --- |
| Obstacle far ahead | HC-SR04 | Prepare to slow, stop or choose a turn |
| Obstacle close | VL53L0X | Stop/turn using the avoidance routine |
| Path clear | Both | Continue the programmed walking gait |

# 4. Features of the Basic Spider Robot

| Feature | How it is achieved |
| --- | --- |
| Four-Leg Walking | Eight MG90S servos provide two controlled joints per leg. |
| Forward Movement | A programmed gait moves all four legs in a coordinated sequence. |
| Backward Movement | The walking sequence can be reversed. |
| Left/Right Turning | Different leg patterns create left and right turns. |
| Standing Position | Servos can move to a predefined stable standing pose. |
| Head Movement | A head servo can be included if the selected mechanical head design supports it. |
| Obstacle Detection | VL53L0X and HC-SR04 detect obstacles in front of the robot. |
| Automatic Obstacle Avoidance | The controller can stop and execute a programmed turn when an obstacle is detected. |
| Expressive Face | The 128×64 OLED acts as the robot's face. |
| Multiple Emotions | The OLED can display happy, sad, angry, surprised, sleepy, normal and warning expressions. |
| Animated Eyes | Eye animations can be used during idle, walking, scanning and obstacle events. |
| Sound Feedback | The buzzer can signal startup, obstacle detection or other robot events. |
| Programmable Gaits | Servo positions can be programmed as movement frames and sequences. |

# 5. Main Controller and Servo System

Arduino Uno is the main controller. The PCA9685 is used because it provides 16 PWM channels, allowing the eight MG90S servos to be controlled without consuming many Arduino I/O resources. Eight servos provide two joints per leg, making the basic quadruped gait possible.

# 6. Expression System

The OLED display is used as the robot's face. Suggested expression states are:

Normal / Idle

Happy

Sad

Angry

Surprised

Sleepy

Thinking / Scanning

Obstacle Warning

Walking

# 7. Power Components

The eight servos should be powered by a properly rated external 5–6 V BEC/buck supply rather than from the Arduino Uno 5 V pin. The battery provides portable power. The 470 µF capacitor is included to help reduce servo-supply voltage fluctuations.

# 8. Components Not Required for This Basic Version

L293D motor driver

L298N motor driver

MPU6050 IMU

DC/BO motors

Camera

ESP32

Additional stabilization sensors

L293D and L298N are DC-motor drivers. The spider's legs use servo motors, so a DC motor driver is not required for this basic design. MPU6050 is also excluded because automatic body stabilization is not part of the basic feature set.

# 9. Final Component Summary

1 × Arduino Uno R3

1 × PCA9685 16-channel servo driver

8 × MG90S servo motors

1 × 128×64 OLED display

1 × VL53L0X ToF sensor

1 × HC-SR04 ultrasonic sensor

1 × 5–6 V high-current BEC/buck converter

1 × rechargeable battery pack

1 × SPST power switch

1 × 470 µF electrolytic capacitor

1 × 5 V buzzer

1 set × 3D-printed body/head/legs

8 × servo mounting brackets

8 × servo horns

4 × robot feet

1 set × mechanical screws/nuts/washers

1 set × spacers/standoffs

# 10. Final Robot Capability

The completed basic robot is a programmable, four-legged walking robot with forward/backward movement, left/right turning, obstacle detection and avoidance, head movement when supported by the mechanical design, an animated OLED face, multiple expressions and buzzer feedback.